PROUDMAN OCEANOGRAPHIC LABORATORY

CRUISE REPORT NO. 40

Inverted Echo Sounders in the Denmark Strait

As part of

FS POSEIDON 290

JUNE 6, 2002 – JUNE 24, 2002

G.W. Hargreaves

2003

DOCUMENT DATA SHEET

| AUTHOR G.W. HARGREAVES | PUBLICATION DATE 2003 |
| --- | --- |

TITLE Inverted Echo Sounders in the Denmark Strait, as part of, FS Poseidon 290, June 6, 2002 – June 24, 2002

REFERENCE Proudman Oceanographic Laboratory, Cruise Report, No 40, 10pp

ABSTRACT The overflow of cold dense water from the Denmark Strait is one of the key elements of the north Atlantic thermohaline circulation and has important consequences for global climate change. It is important to measure the transport of this water and to understand its variability on seasonal and at longer time scales. The European funded project "Variability of Exchanges in Northern Seas" (VEINS MAS3CT960070) was an attempt to measure variations in the Arctic circulation using modern oceanographic instrumentation. The current program is a continuation of this research and may contribute to a proposed new program, Arctic Sub-Arctic Ocean Fluxes (ASOF). A combined Inverted Echo Sounder and Bottom Pressure Recorder was successfully recovered and re-deployed in the Denmark Strait to measure the thickness of this cold dense water and thus determine transport.

| ISSUING ORGANISATION Proudman Oceanographic Laboratory Bidston Observatory Birkenhead Merseyside L43 7RA UK Director: Dr A.E Hill | TELEPHONE: (0151) 653 8633 FAX: (0151) 653 6269 TELEX: 628591 OCEAN BG |
| --- | --- |

| KEYWORDS Bottom Pressure Recorder Denmark Strait Inverted Echo Sounder VEINS Sea Level Bottom Water North Atlantic | CONTRACT PROJECT LT310C PRICE £10.00 |
| --- | --- |

Copies of this report are available from:

The Library, Proudman Oceanographic Laboratory.

CONTENTS

CRUISE PERSONNEL	1

ACKNOWLEDGEMENTS	1

OVERVIEW	1

POL CRUISE OBJECTIVES	2

IES/BPR DEPLOYMENTS	2

Ship Preparation	2

RECOVERY OF IES/BPR (UK1/IES) 13/6/2002	2

IES/BPR (UK1/IES) Recovery Summary	2

DEPLOYMENT OF IES/BPR (UK1/IES) 15/6/2002	3

IES/BPR (UK1/IES) Deployment Information	3

CONCLUSIONS	3

APPENDIX 1 - BPR TECHNICAL INFORMATION	4

IES/BPR (UK1/IES) RECOVERY INFORMATION	4

Logger	4

Inverted Echo Sounder	5

Servicing Information	5

IES/BPR (UK1/IES) DEPLOYMENT INFORMATION	6

MAP OF IES/BPR DEPLOYMENT POSITION	8

GLOSSARY	9

# CRUISE PERSONNEL

POL Personnel

Higher Scientific Officer		Geoff Hargreaves

CEFAS Personnel

Senior Scientific Officer		John Read

IfMH Personnel

Principal Scientific Officer		Jürgen Holfort

Norbert Verch

Ulrich Drübbisch

Markus Janout

Ute Hochbaum

Florian Traum

Filip Hacker

Sönke Heyen

# ACKNOWLEDGEMENTS

The author would like to thank the Captain, Officers and ship's company of FS Poseidon for their help in the recovery and deployment of sea level equipment in the Denmark Strait.

# OVERVIEW

The overflow of cold dense water from the Denmark Strait is one of the key elements of the north Atlantic thermohaline circulation and has important consequences for global climate change.  It is important to measure the transport of this water and to understand its variability on seasonal and at longer time scales.

VEINS (Variability of Exchanges in the Northern Seas) was an EU-MAST project aimed at measuring the variability of ocean fluxes between the Arctic and the North Atlantic for a period of three years. Long-term measurements were made using modern oceanographic instrumentation to determine the variation of the Arctic circulation. Part of this work involves the Denmark Strait where an array of current meters is in place to measure the strength of the Overflow Water (DSOW).  CTD surveys provide knowledge of the physical properties.

To measure its thickness, and hence get a value for transport for the DSOW, an Inverted Echo Sounder was deployed in the core of the current with a view to detecting the echo from the interface between the cold bottom water and the overlying intermediate layer.

The current work program, funded by organisations in the United States of America, is a continuation of the work done during the VEINS project. The data collected may eventually contribute to a proposed new program Arctic Sub-Arctic Ocean Fluxes (ASOF) which is a collaboration between several European countries, the United States and Canada.

# POL CRUISE OBJECTIVES

- To recover one Inverted Echo Sounder in the Denmark Strait.

- To re-deploy one Inverted Echo Sounder in the Denmark Strait.

# IES/BPR DEPLOYMENTS

## Ship Preparation

POL personnel joined FS Poseidon at St Johns, Newfoundland on June 6, 2002. The equipment was loaded aboard the ship, unpacked and stowed safely.

## RECOVERY OF IES/BPR (UK1/IES) 13/6/2002

EVENTS

13.52 GMT			Vessel on station.

13.53 GMT			Release command transmitted.

14.10 GMT			Released from the seabed.

14.47 GMT			On the surface.

Total time on station: 55 minutes

### IES/BPR (UK1/IES) Recovery Summary

Acoustic conditions were very good since the ship shut down the echo sounder and the propellers as soon as it was on station. Communication with both acoustic releases was definite and immediate. The release signal was transmitted to both burnwire release systems and the ship drifted whilst range readings were made to monitor for the moment of release. During ranging, it was clear that the burnwire releases had activated since for every single acoustic ping from the ship, five replies were received. The first reply indicated the range whilst the following four pings verified release activation. Release from the ballast weight was clearly indicated from the slant range reading on the deck unit.

## DEPLOYMENT OF IES/BPR (UK1/IES) 15/6/2002

EVENTS

15.57 GMT		Vessel on station.

15.59 GMT		Release into the water.

16.37 GMT		On the seabed.

Total time on station: 40 minutes.

### IES/BPR (UK1/IES) Deployment Information

The ship was acoustically very quiet, so it was possible to achieve excellent communication with both acoustic releases to the seabed.

## CONCLUSIONS

All of POL cruise objectives were achieved.

# APPENDIX 1 - BPR TECHNICAL INFORMATION

## IES/BPR (UK1/IES) RECOVERY INFORMATION

Location details			-	Latitude	63 28.69' N

Longitude	036 18.81' W

Depth	1991m

On station				-	13.52 GMT on 13/6/2002

Release command transmitted		-	13.53 GMT

Released from seabed			-	14.10 GMT

On surface				-	14.47 GMT

Acoustics fitted were 46457 (Rx 15.0 kHz, Tx 12.0 kHz, Release B) and 46428 (Rx 14.5 kHz, Tx 12.0 kHz, Release D). The release command was transmitted to both Burnwires. Both acoustic units had activated the release and this was indicated by five pings from the acoustic release every time a range reading was taken. Indication of release from the frame was clearly detected by the decreasing slant range readings on the deck unit.

### Logger

Timebase

Expected Scan					Actual Scan

10.00.00 GMT on 14/6/2002			09.58.36 GMT on 14/6/2002

Timebase is 84 seconds fast.

Data were downloaded to uk1bpr0102.raw

Data Arrangement

The raw data are made up of six columns

#### Column					Data

- Scan Time

- Date

- Temperature (DQ 36573)

- Pressure

- Temperature (DQ 38175)

- Pressure

- Blank

- Blank

### Inverted Echo Sounder

IES pinged at 11.07.36 GMT on 14/6/2002.

The data were downloaded to uk1ies0102.v12

Number of data files stored to disk was 4163. The IES was fitted with a Hitachi 1.4Gb disk drive and stored one sample per datafile. Sampling interval was 120 minutes and the sample length was 5409 milliseconds.

Old Batteries

Acoustic release 46457		-	Orange	12.54V

Red	12.56V

Acoustic release 46428		-	Orange 12.35V

Red	12.435V

Burnwire 46457			-	26.6V

Burnwire 46428			-	26.5V

IES logger battery (pack 1)		-	Orange 12.61V

-	Red 	12.56V

IES logger battery (pack 2)		-	Orange 12.17V

-	Red 	12.16V

IES logger battery (pack 3)		-	Orange 12.47V

-	Red 	12.48V

IES transponder battery (pack 4)	-	Orange	12.13V

-	Red	12.13V

### Servicing Information

The frame is of the MkIV variety and is in good condition with little sign of corrosion. The BPR logger tube was in good condition, if slightly dirty looking. The EPROM card was removed for data retrieval and the EPROMs then erased. Both stainless steel clamps of the Inverted Echo Sounder were corroded and these were replaced by a single new clamp. The condition of the glass sphere is giving cause for concern. There are thin fragments of glass breaking off from near to the sealing face on the inside of the sphere. It should survive for another deployment but must be replaced next year.

## IES/BPR (UK1/IES) DEPLOYMENT INFORMATION

Location details			-	Latitude	63 28.58' N

Longitude	036 17.37' W

Depth	1991m

On station				-	15.57 GMT on 15/6/2002

Released into the water		-	15.58 GMT

On seabed				-	16.37 GMT

Acoustic Information

Benthos XT6000 46428		-	Rx 14.5 kHz, Tx 12.0 kHz, Release D

Benthos XT6000 46457		-	Rx 15.0 kHz, Tx 12.0 kHz, Release B

Both of the acoustic units are using a burnwire release mechanism.

Logger

Logger	SSDL 5 with sensors DQ 36573 and DQ 38175

Timebase started 11.00.00 GMT on 14/6/2002

First scan at 11.15.00 GMT on 14/6/2002

Inverted Echo Sounder

IES fitted with POL ADC board and Hitachi 1.4GB disk drive.

IES started at 09.59.40 GMT on 15/6/2002.

First chirp at 11.59.59 GMT on 15/6/2002.

Set-up parameters

Chirp interval				-	120 minutes

Sample rate				-	fast

Samples per datafile			-	1

Lockout (1/100s)			-	0

Start file number			-	1

Serial number				-	5

Deployment number			-	8

Comment				-	UK1/IES0203 Denmark Strait

Recovery Equipment

Benthos Radio Beacon			-	154.585 MHz, 	Channel A

New Battery Information

Acoustic release 46457		-	Orange	14.51V

Red	14.51V

Acoustic release 46428		-	Orange 14.18V

Red	14.19V

Burnwire 46457			-	28.6V

Burnwire 46428			-	28.6V

IES logger battery (pack 1)		-	Orange 14.11V

-	Red 	14.18V

IES logger battery (pack 2)		-	Orange 14.17V

-	Red 	14.18V

IES logger battery (pack 3)		-	Orange 14.15V

-	Red 	14.17V

IES transponder battery (pack 4)	-	Orange	14.18V

-	Red	14.18V

# MAP OF IES/BPR DEPLOYMENT POSITION

# GLOSSARY

ADC		-	Analogue to Digital Converter

ASOF		-	Artic Sub-Arctic Ocean Fluxes

BPR		-	Bottom Pressure Recorder

CEFAS	-	Centre for the Environment, Fisheries and Aquaculture Science

CTD		-	Conductivity, Temperature and Depth Profiler

DSOW		-	Denmark Strait Overflow Water

EPROM	-	Erasable Programmable Memory

GMT		-	Greenwich Mean Time

IES		-	Inverted Echo Sounder

IfMH		-	Institut für Meereskunde, Hamburg University

POL		-	Proudman Oceanographic Laboratory

VEINS		-	Variability of Exchanges in Northern Seas